## Part A — What you need

### Software

- VirtualBox or VMware Workstation Player

- Windows Server 2019/2022 ISO (Evaluation is fine)

- Windows 10/11 ISO (for client)

- Cisco Packet Tracer (the latest you have)

### Lab goal (what you’ll end up with)

- 1x Domain Controller (AD DS + DNS) on Windows Server

- 1–2x Domain-joined clients (Windows 10/11)

- A routed LAN topology in Packet Tracer (optionally VLANs)

- Working:

  - clients get IPs (DHCP)

  - clients use DC for DNS

  - clients join the domain

  - users/computers/groups/GPO work

## Part B — Build the Active Directory / Domain Controller lab (VMs)

### Step 1 — Create the Domain Controller VM

- Create a VM named: DC01

- Allocate (recommended):

  - CPU: 2 cores

  - RAM: 4–8 GB

  - Disk: 60 GB

- Network adapter:

  - If you want multi-VM lab only: Host-Only or Internal Network

  - If you also want internet: add 2nd adapter as NAT (optional)

### Step 2 — Install Windows Server

- Boot the VM from the Windows Server ISO

- Install Windows Server Standard (Desktop Experience)

- Set local admin password

- Log in

### Step 3 — Set a static IP on DC01 (required)

- Open: Control Panel → Network and Internet → Network Connections

- Right click Ethernet → Properties → IPv4

- Set:

  - IP: 192.168.10.10

  - Mask: 255.255.255.0

  - Gateway: 192.168.10.1 (you’ll use this if you have a router; otherwise can be blank for simple host-only)

  - DNS: 192.168.10.10 (the DC points to itself)

### Step 4 — Rename server and reboot

- Server Manager → Local Server → Computer name → Change

- Rename to DC01

- Reboot

### Step 5 — Add AD DS role + DNS role

- Server Manager → Add roles and features

- Role-based installation → choose DC01

- Select:

  - Active Directory Domain Services

  - It will prompt for features → Add

  - Also select DNS Server (commonly installed with AD)

- Install

### Step 6 — Promote to Domain Controller (create a new forest)

- In Server Manager, click the flag: Promote this server to a domain controller

- Choose: Add a new forest

- Domain name (example): lab.local

- Set DSRM password (write it down)

- Leave defaults for DNS delegation

- Install → it reboots

### Step 7 — Verify AD + DNS

After reboot:

- Open Active Directory Users and Computers (ADUC)

- Open DNS Manager

- Confirm you see:

  - forward lookup zone: lab.local

  - _msdcs zone entries exist (normal)

## Part C — Create client VM(s) and join to the domain

### Step 8 — Create a Windows 10/11 client VM

- Create VM: PC01

- 2 cores, 4 GB RAM, 60 GB disk (or less)

- Put it on the same VM network as DC01 (same “Internal Network” name or same Host-Only network)

- Install Windows 10/11

### Step 9 — Configure client IP + DNS (key step)

On PC01:

- Set IPv4 to either DHCP (if you later add DHCP) or static for now:

  - IP: 192.168.10.20

  - Mask: 255.255.255.0

  - Gateway: 192.168.10.1 (optional if no router)

  - DNS: 192.168.10.10 (must point to DC)

### Step 10 — Test connectivity

On PC01 (Command Prompt):

- ping 192.168.10.10

- nslookup lab.local
If DNS fails, your DNS setting is wrong (most common mistake).

### Step 11 — Join domain

- Settings → System → About → Domain or workgroup

- Join domain: lab.local

- Enter domain admin creds: LAB\Administrator (or Administrator depending)

- Reboot

- Log in as: LAB\Administrator or a domain user you create

## Part D — Add DHCP (optional but very common in labs)

You can do DHCP on the Windows Server, or on a router. In AD labs, doing it on the server is common.

### Step 12 — Install DHCP Server role on DC01

- Server Manager → Add roles → DHCP Server

- Install → then Complete DHCP configuration

### Step 13 — Create a DHCP scope

- Open DHCP console

- IPv4 → New Scope

- Example scope:

  - Range: 192.168.10.100 – 192.168.10.200

  - Mask: 255.255.255.0

  - Gateway option (003): 192.168.10.1 (if you have routing)

  - DNS option (006): 192.168.10.10

  - Domain option (015): lab.local

- Activate scope

### Step 14 — Set clients to DHCP

On PC01: set NIC to DHCP, renew:

- ipconfig /release

- ipconfig /renew

## Part E — Do core AD tasks (users, groups, OUs, GPO)

### Step 15 — Create OUs

In ADUC:

- Right click domain → New → Organizational Unit

- Create:

  - OU=Users

  - OU=Computers

  - OU=Admins (optional)

### Step 16 — Create users and groups

- In Users OU: New → User

  - create jdoe, password

- Create groups:

  - GG-IT-Admins

  - GG-Staff

- Add members accordingly

### Step 17 — Create and test a GPO

- Open Group Policy Management

- Right click Users OU → Create GPO

- Example easy lab policy:

  - User Configuration → Administrative Templates → Control Panel

  - “Prohibit access to Control Panel and PC settings” → Enabled

- On client:

  - log in as jdoe

  - run: gpupdate /force

  - verify Control Panel is blocked

# Part F — Cisco Packet Tracer side (network lab that supports AD)

Since Packet Tracer can’t host AD, your Packet Tracer topology is your network simulation: VLANs, routing, DHCP relay, access control. You validate that a “DC subnet” and “client subnet(s)” can reach each other and that DNS/DHCP traffic would work.

## Topology (recommended)

- 1 Router (2911) or L3 Switch (3560)

- 1 Switch (2960)

- 2 VLANs:

  - VLAN 10 = Servers 192.168.10.0/24

  - VLAN 20 = Clients 192.168.20.0/24

## Step 18 — Build VLANs on the switch (2960)

On the switch:

- Create VLANs:

  - VLAN 10 name SERVERS

  - VLAN 20 name CLIENTS

- Put ports into VLANs:

  - Port to “server PC” in PT → access vlan 10

  - Port to “client PC” in PT → access vlan 20

- Trunk uplink to router/L3 switch:

  - set trunk allowed VLANs 10,20

## Step 19 — Router-on-a-stick (if using a router)

On router:

- On interface connected to switch (e.g. G0/0):

  - Create subinterfaces:

    - G0/0.10 encaps dot1q 10, IP 192.168.10.1/24

    - G0/0.20 encaps dot1q 20, IP 192.168.20.1/24

## Step 20 — DHCP in PT (choose one)

### Option A: DHCP on router

- Exclude addresses:

  - 192.168.10.1-10.20

  - 192.168.20.1-20.20

- Create pools:

  - SERVER pool (VLAN10): default-router 192.168.10.1, dns-server 192.168.10.10

  - CLIENT pool (VLAN20): default-router 192.168.20.1, dns-server 192.168.10.10

- If you want DHCP server centralized in VLAN10 only, you’d use ip helper-address on VLAN20 interface pointing to DHCP server (that’s a common real design).

### Option B: DHCP server device in PT

- Add PT “Server” device in VLAN10

- Configure DHCP service there

- On router subinterface VLAN20: ip helper-address 192.168.10.x

## Step 21 — DNS in PT (simulate DC DNS)

In Packet Tracer you can:

- Point clients’ DNS to the PT “Server” device

- Or just validate Layer 3 reachability to where DNS would live (192.168.10.10)

## Step 22 — Connectivity tests in Packet Tracer

From client PC:

- Ping default gateway

- Ping server subnet gateway

- Ping “DNS/DC address”

- If using PT DNS service: try browsing to a hostname you created

# Part G — Tie the two together (how to make it feel like one lab)

You have two practical options:

### Option 1 (most common for students)

- Packet Tracer = network design + validation

- VMs = real AD
You document: addressing/VLANs/routing/DHCP relay and show that the VM design matches it.

### Option 2 (more advanced)

Use a real virtual network (GNS3/EVE-NG or real router) so your VM clients actually traverse VLANs. Packet Tracer can’t connect to your VM traffic directly in a realistic way.

## Common mistakes checklist (so you don’t get stuck)

- Client DNS is not pointing to DC (192.168.10.10) → domain join fails

- DC doesn’t have a static IP → AD/DNS gets flaky

- Time difference between client and DC is too large → login/auth issues

- DHCP hands out wrong DNS server → clients can’t find domain

- VLAN trunk not set → inter-VLAN routing fails

- Missing ip helper-address when DHCP server is in another subnet